Original: English

FTAA - JOINT GOVERNMENT-PRIVATE SECTOR COMMITTEE OF EXPERTS ON ELECTRONIC COMMERCE

UNITED STATES

NEW AND DEVELOPING INTERNET ACCESS ALTERNATIVES

With input from Private Sector and Government Representatives

The following document is intended to illustrate the variety of technologies that will impact access and infrastructure issues related to e-commerce.  It is not intended to be an exhaustive treatment of the subject.

New and Developing Internet Access Alternatives

There are a growing number of options for accessing the Internet including wireless, cable modems, and new high-bandwidth wireline applications.   These are all of interest and will provide for access by a variety of means including TV and cell phones that do not necessarily depend on the public switched telephone network (PSTN).  At least one estimate indicates that by 2002, there will be 30 million Internet users in Latin America with the vast majority of these users not utilizing traditional PC’s.

In order to look at these potential access methods we can distinguish them as wireless or wired and explore various technologies and applications available under each.

Wireless Telephony

1. Mobile Wireless Telephony

British Telecom expects the number of global wireless subscribers to jump 200 percent in the next five years to more than 1 billion, with worldwide mobile e-commerce growing to $200 billion in four years.  This gives an idea of the expected market for mobile e-commerce.

Wireless subscribers seeking access to the Internet are beginning to take advantage of an interesting new product now available for cellular phones -- Wireless Access Protocol (WAP).  WAP enables web surfing through a standard cellular connection. WAP technology is now available around the world.  For example, a cellular provider in Brazil just announced that it was purchasing two million WAP-enabled handsets to distribute this year.

While the current cellular networks, (so-called second generation or 2G systems) are not capable of transmitting data at much more than 14.4 kilobytes per second (Kbps), third generation systems (3G) and the intermediary systems will be capable of much more in the not-so-distant future.  3G systems are scheduled to give a data rate of 2 megabytes per second (Mbps). In fact, they will be able to provide even more. While this is still somewhat in the future, it is clear that a relatively inexpensive cellular phone will be able to facilitate many Internet functions and operations.

Several software products are starting to appear to assist moving about the Internet with a device that has a very simple user interface. These include programs allowing very efficient synchronization, which means that a keyboard-equipped device can be used to input the data most utilized. Likewise, we are now seeing voice servers appear on the market that utilize the programming language VoxML.  As voice is the most sophisticated input scheme available to a cellular telephone, we can expect that these servers will become very important as adjuncts to the normal servers on the Internet.

In addition to mobile wireless telephony, the chart below outlines several other interesting possibilities for providing access to the Internet. These are subsequently discussed.

Alternative Broadband Telecommunications

and Internet Access Technologies

| Technology | Advantages | Disadvantages |
| --- | --- | --- |
| Wireless |  |  |

| LMDS | - AAmple Spectrum | - RRain, Line of Sight (LOS) Issues - CCost of Radio |
| --- | --- | --- |

| - MMDS | - EExisting Technology - HHigh Capacity | - MMultiple path LOS required (Hub/Spoke) - AAll subscriber equipment in common equipment space. |
| --- | --- | --- |
| Wired |  |  |

| - WWholesale T1 | - WWell-Known Technology | - MMust be built for peak loads - RResources not shared/Inefficient use of medium. |
| --- | --- | --- |

| - DSL | - IInternational success (better line quality) - SSubstantial penetration of copper infrastructure - PPowerful “incumbent” operators to push technology and services | - OOngoing Technical Problems: Installation, Interference, Unresolved Standards, Performance - CCustomer Confusion (many “flavors”) - RRequires line conditioning - LLines unusable or nonexistent - LLeased-line profit erosion (“slow-roll”) |
| --- | --- | --- |

| - CCable Modems | - CConnection-Oriented (“Always On”) - SShared Cost attributes - Band width Flexibility - PPrivately-managed Backbones - SSystem Robustness | - 22-Way Cable Infrastructure not fully built-out - EExisting Services and Infrastructure Residential-focused - CCapital Requirements & Debt-load of Industry |
| --- | --- | --- |

| - FFiber | - RReliability - CCapacity - MManageability (Predictability) | - HHigh Cost of Deployment per Subscriber - RRestricted Accessibility to Install - MMajor City and Backbone Prevalent |
| --- | --- | --- |

2. Local Multipoint Distribution Service (LMDS)

LMDS is a fixed wireless technology capable of providing broadband service.  Because of the large amount of spectrum available at the frequencies millimeter wave technology operates, it has the capability of providing “fiber-like” speeds.  However, because LMDS operates at such high frequencies (30GHz) and requires line-of-site towers, its range is typically no more than three or four miles.  Thus, its most common use is as a fiber extender as the commercial last-kilometer for high-speed networking.  In other words, LMDS is the final link to points of services such as businesses and homes.  Although building an LMDS system is far less expensive than deploying new wireline facilities, the current price for LMDS is such that it will be some time before it is available at a cost that might attract individual consumer users. The major deployment of LMDS is foreseen to be as a E1/T1 carrier rather than as a data centric deployment.

3. Multipoint Multichannel Distribution System (MMDS)

MMDS is another fixed wireless technology that offers alternative means of extending a telecommunications network and expanding Internet access for rural users.  MMDS operates in 2.5GHz, lower than LMDS, which allows less expensive radio and network design because strict line-of-site construction is not required.  MMDS signals also travel longer distances than LMDS  – up to 35 miles under the best circumstances.  MMDS is expected to be available at a price low enough to be attained by high-end consumers and SMEs.  Two-way MMDS technology is relatively new in the United States and further standards work is now going on in the “802 Committee” of the Institute of Electrical and Electronics Engineers (IEEE) to standardize a method of providing both data centric and voice centric transmissions on the same devices.

4. UII (Not shown on chart)

UII is a band set aside in the United States for low cost wireless access to the Internet. This band in the 5GHz range has spawned several product offerings, which provide data centric access. Because they are designed for low cost access, most have lower reliability than the more robust solutions described above. However, for circumstances in which high speed access is desired, and somewhat diminished reliability can be tolerated, the UII-band may be a very low cost solution.

Wired Telephony

In addition to new wireless technologies, there are a number of wired technologies that offer alternative means of access to the Internet and telecommunications services.  Wholesale E1/T1 and Fiber are the traditional means of providing high speed telecommunications and data access, though these technologies have traditionally been used to facilitate corporate access rather than residential access.  Two newer technologies, DSL and Cable Modems, may be more likely adopted for the residential marketplace.

5. Digital Subscriber Line (DSL)

DSL is an alternative broadband service whose deployment is growing rapidly in the United States.  According to TeleChoice, approximately one third of DSL services are business at this time.  However, longer term estimates from the Yankee Group suggest that there may be as many as 7 million DSL customers in 2004.

The customer start-up cost of DSL is about $200 to $300.  Unlike cable modems, whose performance varies with the number of users on the system at any time, DSL broadband operates at a fixed rate between 250 and 600 kilobits/second. DSL technology does not generally require extensive upgrades of equipment.  Instead, most standard copper lines can be made into DSL lines by installing electronic equipment on either end of the loop.

However, there are some telephone lines that are not DSL-capable. This is because telephone loops in some areas extend over 18,000 feet (about 3 ½ miles) from their central switching offices.  Loops of that length require additional “loading” (electrical inductors) to maintain the quality of the traditional voice service and these inductors tend to block the high frequencies required for DSL service.  Unlike the mobile wireless telephony and cable modems, it is necessary to have a computing device for DSL service. The major weakness of DSL in developing countries is that often the copper infrastructure is not adequate to reach all of the population.

6. Cable Modems

Cable modems utilize a cable television network that has been upgraded to provide two-way  communication.  Cable modems offer broadband Internet access capability to residential subscribers.  However, Internet speed deteriorates as more users on the cable system subscribe to broadband Internet service.  Cable modems do not necessarily utilize the traditional PC.  Subscribers often use relatively inexpensive “set top” boxes, which essentially turn the TV set into an Internet browser.  It appears that these devices, which cost the cable provider about US$150, may be leased to the ultimate consumer for a few dollars a month.  Like cellular phones, cable modems have the capability of substantially lowering the cost of Internet access for the consumer.

Use of Wireless Technology in Expanding Access in the U.S.

Some Examples

There are many examples of successful use of wireless technology to expand access in the United States.  Below are several examples.

Southwest Navajo Nation Virtual Alliance

The Southwest Navajo Nation in Arizona is creating a wireless community network to link five local government entities, called chapters.  The chapters are spread over a 5,000 square mile region in southwest Arizona that suffers severe economic, ecological, and health problems.  The average telephone penetration for the Navajo Reservation is only 19 percent.  The system will use very small aperture terminals (VSAT) Internet access systems and kiosk touch-screen work stations to deliver interactive, community networking and public service-related resources in both English and Navajo languages.  Services provided will include facilitating access to loans providing health and public safety alerts, maintaining Navajo language and cultural resources.  Voice files will be available so users can listen to contents in the oral Navajo language.

Sevier River Water Users Association

The Sevier River Water Users Association is developing a water management network that will improve water management, water conservation, and result in better crop yields throughought the Sevier River Basin in central Utah.  The economy of this large, rural area is heavily reliant on farming.  Real-time data collection from remote parts of the watershed will be possible using both wireline and wireless connections.

Columbia-Presbyterian Medical Center

Colombia-Presbyterian Medical Center uses wireless, hand-held computers to coordinate and manage the care of tuberculosis patients in New York City.

Muncie Public Library Cybermobile

The Public Library in Muncie, Indiana has built a Cybermobile that provides free computer and Internet training courses for seniors, people in correctional facilities, the physically and mentally challenged, immigrants learning English, adult literacy groups, and others.  By using VSAT Internet access technology, the bus has expanded Internet access in Muncie and has provided basic computer literacy training for hundreds.

Western Wireless (WW)

WW uses advanced mobile phone system (AMPS) wireless local loop (WLL) technology to serve traditionally underserved Indian Reservations.

CenturyTel

CenturyTel offers competitive local service using time division multiple access (TDMA) technology to offer a version of WLL that allows a specially-made phone to operate as a traditional wireline phone at home and a wireless phone away from home (up to 7 miles away).

Northern Cheyenne and Crow Indian Health Care Services

Health care professionals are now using a device first developed by NASA to monitor the deterioration of astronaut’s eyeballs during spaceflights to remotely examine the eyes of Northern Cheyenne and Crow Indian people who suffer from a high rate of diabetes.  The device enables the wireless transfer of medical data while the patients are in their own homes rather than having to travel for hours to the nearest medical facility for regular check-ups.  Only 20 percent of these Native Americans typically have access to a telephone.